SYNCR Complete Modbus Register Specification

SYNCR v2.6.6  |  Total 188 Defined Registers (163 Growatt MAX + 25 Growatt MIN)

1. Growatt MAX 150KTL3-X: Solar DC & Primary Status (FC04 - 8 Registers)

| Address | Parameter Name | Function Code | Data Type | Scale | Unit | Plotted |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | Inverter Status | FC04 | UInt16 | 1.0 | — | Yes |
| 1 | PV Input Power | FC04 | UInt32 | 0.1 | W | Yes |
| 3 | PV1 Voltage | FC04 | UInt16 | 0.1 | VDC | Yes |
| 4 | PV1 Current | FC04 | UInt16 | 0.1 | A | Yes |
| 5 | PV1 Power | FC04 | UInt32 | 0.1 | W | Yes |
| 7 | PV2 Voltage | FC04 | UInt16 | 0.1 | VDC | Yes |
| 8 | PV2 Current | FC04 | UInt16 | 0.1 | A | Yes |
| 9 | PV2 Power | FC04 | UInt32 | 0.1 | W | Yes |

2. Growatt MAX 150KTL3-X: 3-Phase Grid AC Output (FC04 - 14 Registers)

| Address | Parameter Name | Function Code | Data Type | Scale | Unit | Plotted |
| --- | --- | --- | --- | --- | --- | --- |
| 35 | AC Output Power | FC04 | UInt32 | 0.1 | W | Yes |
| 37 | Grid Frequency | FC04 | UInt16 | 0.01 | Hz | Yes |
| 38 | Grid Voltage Vac1 (Phase 1) | FC04 | UInt16 | 0.1 | VAC | Yes |
| 39 | Grid Current Iac1 (Phase 1) | FC04 | UInt16 | 0.1 | A | Yes |
| 40 | Phase 1 Power | FC04 | UInt32 | 0.1 | W | Yes |
| 42 | Grid Voltage Vac2 (Phase 2) | FC04 | UInt16 | 0.1 | VAC | Yes |
| 43 | Grid Current Iac2 (Phase 2) | FC04 | UInt16 | 0.1 | A | Yes |
| 44 | Phase 2 Power | FC04 | UInt32 | 0.1 | W | Yes |
| 46 | Grid Voltage Vac3 (Phase 3) | FC04 | UInt16 | 0.1 | VAC | Yes |
| 47 | Grid Current Iac3 (Phase 3) | FC04 | UInt16 | 0.1 | A | Yes |
| 48 | Phase 3 Power | FC04 | UInt32 | 0.1 | W | Yes |
| 50 | Line Voltage Vac_RS | FC04 | UInt16 | 0.1 | VAC | No |
| 51 | Line Voltage Vac_ST | FC04 | UInt16 | 0.1 | VAC | No |
| 52 | Line Voltage Vac_TR | FC04 | UInt16 | 0.1 | VAC | No |

3. Growatt MAX 150KTL3-X: Energy Accumulation (FC04 - 3 Registers)

| Address | Parameter Name | Function Code | Data Type | Scale | Unit | Plotted |
| --- | --- | --- | --- | --- | --- | --- |
| 53 | Energy Today | FC04 | UInt32 | 0.1 | kWh | Yes |
| 55 | Energy Total | FC04 | UInt32 | 0.1 | kWh | Yes |
| 57 | Work Time Total | FC04 | UInt32 | 0.5 | h | No |

4. Growatt MAX 150KTL3-X: Diagnostics & Temperatures (FC04 - 10 Registers)

| Address | Parameter Name | Function Code | Data Type | Scale | Unit | Plotted |
| --- | --- | --- | --- | --- | --- | --- |
| 93 | Inverter Temperature | FC04 | UInt16 | 0.1 | °C | Yes |
| 94 | IPM Temperature | FC04 | UInt16 | 0.1 | °C | Yes |
| 95 | Boost Temperature | FC04 | UInt16 | 0.1 | °C | No |
| 98 | P Bus Voltage | FC04 | UInt16 | 0.1 | V | Yes |
| 99 | N Bus Voltage | FC04 | UInt16 | 0.1 | V | Yes |
| 100 | Power Factor | FC04 | UInt16 | 0.001 | — | No |
| 101 | Output Power % | FC04 | UInt16 | 1.0 | % | Yes |
| 104 | Derating Mode | FC04 | UInt16 | 1.0 | — | No |
| 105 | Fault Code | FC04 | UInt16 | 1.0 | — | No |
| 110 | Warning Bitmask | FC04 | UInt32 | 1.0 | — | No |

5. Growatt MAX 150KTL3-X: Solar String Monitoring (FC04 - 32 Registers)

| Address | Parameter Name | Function Code | Data Type | Scale | Unit | Plotted |
| --- | --- | --- | --- | --- | --- | --- |
| 141 | String 1 Voltage | FC04 | UInt16 | 0.1 | VDC | Yes |
| 142 | String 1 Current | FC04 | Int16 | 0.1 | A | Yes |
| 143 | String 2 Voltage | FC04 | UInt16 | 0.1 | VDC | Yes |
| 144 | String 2 Current | FC04 | Int16 | 0.1 | A | Yes |
| 145 | String 3 Voltage | FC04 | UInt16 | 0.1 | VDC | Yes |
| 146 | String 3 Current | FC04 | Int16 | 0.1 | A | Yes |
| 147 | String 4 Voltage | FC04 | UInt16 | 0.1 | VDC | Yes |
| 148 | String 4 Current | FC04 | Int16 | 0.1 | A | Yes |
| 149 | String 5 Voltage | FC04 | UInt16 | 0.1 | VDC | No |
| 150 | String 5 Current | FC04 | Int16 | 0.1 | A | No |
| 151 | String 6 Voltage | FC04 | UInt16 | 0.1 | VDC | No |
| 152 | String 6 Current | FC04 | Int16 | 0.1 | A | No |
| 153 | String 7 Voltage | FC04 | UInt16 | 0.1 | VDC | No |
| 154 | String 7 Current | FC04 | Int16 | 0.1 | A | No |
| 155 | String 8 Voltage | FC04 | UInt16 | 0.1 | VDC | No |
| 156 | String 8 Current | FC04 | Int16 | 0.1 | A | No |
| 157 | String 9 Voltage | FC04 | UInt16 | 0.1 | VDC | No |
| 158 | String 9 Current | FC04 | Int16 | 0.1 | A | No |
| 159 | String 10 Voltage | FC04 | UInt16 | 0.1 | VDC | No |
| 160 | String 10 Current | FC04 | Int16 | 0.1 | A | No |
| 161 | String 11 Voltage | FC04 | UInt16 | 0.1 | VDC | No |
| 162 | String 11 Current | FC04 | Int16 | 0.1 | A | No |
| 163 | String 12 Voltage | FC04 | UInt16 | 0.1 | VDC | No |
| 164 | String 12 Current | FC04 | Int16 | 0.1 | A | No |
| 165 | String 13 Voltage | FC04 | UInt16 | 0.1 | VDC | No |
| 166 | String 13 Current | FC04 | Int16 | 0.1 | A | No |
| 167 | String 14 Voltage | FC04 | UInt16 | 0.1 | VDC | No |
| 168 | String 14 Current | FC04 | Int16 | 0.1 | A | No |
| 169 | String 15 Voltage | FC04 | UInt16 | 0.1 | VDC | No |
| 170 | String 15 Current | FC04 | Int16 | 0.1 | A | No |
| 171 | String 16 Voltage | FC04 | UInt16 | 0.1 | VDC | No |
| 172 | String 16 Current | FC04 | Int16 | 0.1 | A | No |

6. Growatt MAX 150KTL3-X: Holding Registers (FC03 - 9 Registers)

| Address | Parameter Name | Function Code | Data Type | Scale | Unit | Plotted |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | Remote On/Off Control | FC03 | UInt16 | 1.0 | — | No |
| 3 | Active Power Rate % | FC03 | UInt16 | 1.0 | % | No |
| 4 | Reactive Power Rate % | FC03 | UInt16 | 1.0 | % | No |
| 22 | Baud Rate Select | FC03 | UInt16 | 1.0 | — | No |
| 24 | Inverter Serial Number | FC03 | String8 | 1.0 | — | No |
| 30 | Comm Address (Slave ID) | FC03 | UInt16 | 1.0 | — | No |
| 43 | Device Type Code | FC03 | UInt16 | 1.0 | — | No |
| 44 | Tracker / Phase Config | FC03 | UInt16 | 1.0 | — | No |
| 45 | System Date / Time | FC03 | UInt16 | 1.0 | — | No |

7. Growatt MAX 150KTL3-X: Diagnostic Metrics 1 to 25 (FC04 - 25 Registers)

| Address | Parameter Name | Function Code | Data Type | Scale | Unit | Plotted |
| --- | --- | --- | --- | --- | --- | --- |
| 180 | Diag Metric 1 | FC04 | UInt32 | 0.1 | pts | No |
| 182 | Diag Metric 2 | FC04 | UInt32 | 0.1 | pts | No |
| 184 | Diag Metric 3 | FC04 | UInt32 | 0.1 | pts | No |
| 186 | Diag Metric 4 | FC04 | UInt32 | 0.1 | pts | No |
| 188 | Diag Metric 5 | FC04 | UInt32 | 0.1 | pts | No |
| 190 | Diag Metric 6 | FC04 | UInt32 | 0.1 | pts | No |
| 192 | Diag Metric 7 | FC04 | UInt32 | 0.1 | pts | No |
| 194 | Diag Metric 8 | FC04 | UInt32 | 0.1 | pts | No |
| 196 | Diag Metric 9 | FC04 | UInt32 | 0.1 | pts | No |
| 198 | Diag Metric 10 | FC04 | UInt32 | 0.1 | pts | No |
| 200 | Diag Metric 11 | FC04 | UInt32 | 0.1 | pts | No |
| 202 | Diag Metric 12 | FC04 | UInt32 | 0.1 | pts | No |
| 204 | Diag Metric 13 | FC04 | UInt32 | 0.1 | pts | No |
| 206 | Diag Metric 14 | FC04 | UInt32 | 0.1 | pts | No |
| 208 | Diag Metric 15 | FC04 | UInt32 | 0.1 | pts | No |
| 210 | Diag Metric 16 | FC04 | UInt32 | 0.1 | pts | No |
| 212 | Diag Metric 17 | FC04 | UInt32 | 0.1 | pts | No |
| 214 | Diag Metric 18 | FC04 | UInt32 | 0.1 | pts | No |
| 216 | Diag Metric 19 | FC04 | UInt32 | 0.1 | pts | No |
| 218 | Diag Metric 20 | FC04 | UInt32 | 0.1 | pts | No |
| 220 | Diag Metric 21 | FC04 | UInt32 | 0.1 | pts | No |
| 222 | Diag Metric 22 | FC04 | UInt32 | 0.1 | pts | No |
| 224 | Diag Metric 23 | FC04 | UInt32 | 0.1 | pts | No |
| 226 | Diag Metric 24 | FC04 | UInt32 | 0.1 | pts | No |
| 228 | Diag Metric 25 | FC04 | UInt32 | 0.1 | pts | No |

8. Growatt MAX 150KTL3-X: Extended Registers 1 to 62 (FC04 - 62 Registers)

| Address | Parameter Name | Function Code | Data Type | Scale | Unit | Plotted |
| --- | --- | --- | --- | --- | --- | --- |
| 302 | Extended Register 1 | FC04 | UInt32 | 0.1 | — | No |
| 304 | Extended Register 2 | FC04 | UInt32 | 0.1 | — | No |
| 306 | Extended Register 3 | FC04 | UInt32 | 0.1 | — | No |
| 308 | Extended Register 4 | FC04 | UInt32 | 0.1 | — | No |
| 310 | Extended Register 5 | FC04 | UInt32 | 0.1 | — | No |
| 312 | Extended Register 6 | FC04 | UInt32 | 0.1 | — | No |
| 314 | Extended Register 7 | FC04 | UInt32 | 0.1 | — | No |
| 316 | Extended Register 8 | FC04 | UInt32 | 0.1 | — | No |
| 318 | Extended Register 9 | FC04 | UInt32 | 0.1 | — | No |
| 320 | Extended Register 10 | FC04 | UInt32 | 0.1 | — | No |
| 322 | Extended Register 11 | FC04 | UInt32 | 0.1 | — | No |
| 324 | Extended Register 12 | FC04 | UInt32 | 0.1 | — | No |
| 326 | Extended Register 13 | FC04 | UInt32 | 0.1 | — | No |
| 328 | Extended Register 14 | FC04 | UInt32 | 0.1 | — | No |
| 330 | Extended Register 15 | FC04 | UInt32 | 0.1 | — | No |
| 332 | Extended Register 16 | FC04 | UInt32 | 0.1 | — | No |
| 334 | Extended Register 17 | FC04 | UInt32 | 0.1 | — | No |
| 336 | Extended Register 18 | FC04 | UInt32 | 0.1 | — | No |
| 338 | Extended Register 19 | FC04 | UInt32 | 0.1 | — | No |
| 340 | Extended Register 20 | FC04 | UInt32 | 0.1 | — | No |
| 342 | Extended Register 21 | FC04 | UInt32 | 0.1 | — | No |
| 344 | Extended Register 22 | FC04 | UInt32 | 0.1 | — | No |
| 346 | Extended Register 23 | FC04 | UInt32 | 0.1 | — | No |
| 348 | Extended Register 24 | FC04 | UInt32 | 0.1 | — | No |
| 350 | Extended Register 25 | FC04 | UInt32 | 0.1 | — | No |
| 352 | Extended Register 26 | FC04 | UInt32 | 0.1 | — | No |
| 354 | Extended Register 27 | FC04 | UInt32 | 0.1 | — | No |
| 356 | Extended Register 28 | FC04 | UInt32 | 0.1 | — | No |
| 358 | Extended Register 29 | FC04 | UInt32 | 0.1 | — | No |
| 360 | Extended Register 30 | FC04 | UInt32 | 0.1 | — | No |
| 362 | Extended Register 31 | FC04 | UInt32 | 0.1 | — | No |
| 364 | Extended Register 32 | FC04 | UInt32 | 0.1 | — | No |
| 366 | Extended Register 33 | FC04 | UInt32 | 0.1 | — | No |
| 368 | Extended Register 34 | FC04 | UInt32 | 0.1 | — | No |
| 370 | Extended Register 35 | FC04 | UInt32 | 0.1 | — | No |
| 372 | Extended Register 36 | FC04 | UInt32 | 0.1 | — | No |
| 374 | Extended Register 37 | FC04 | UInt32 | 0.1 | — | No |
| 376 | Extended Register 38 | FC04 | UInt32 | 0.1 | — | No |
| 378 | Extended Register 39 | FC04 | UInt32 | 0.1 | — | No |
| 380 | Extended Register 40 | FC04 | UInt32 | 0.1 | — | No |
| 382 | Extended Register 41 | FC04 | UInt32 | 0.1 | — | No |
| 384 | Extended Register 42 | FC04 | UInt32 | 0.1 | — | No |
| 386 | Extended Register 43 | FC04 | UInt32 | 0.1 | — | No |
| 388 | Extended Register 44 | FC04 | UInt32 | 0.1 | — | No |
| 390 | Extended Register 45 | FC04 | UInt32 | 0.1 | — | No |
| 392 | Extended Register 46 | FC04 | UInt32 | 0.1 | — | No |
| 394 | Extended Register 47 | FC04 | UInt32 | 0.1 | — | No |
| 396 | Extended Register 48 | FC04 | UInt32 | 0.1 | — | No |
| 398 | Extended Register 49 | FC04 | UInt32 | 0.1 | — | No |
| 400 | Extended Register 50 | FC04 | UInt32 | 0.1 | — | No |
| 402 | Extended Register 51 | FC04 | UInt32 | 0.1 | — | No |
| 404 | Extended Register 52 | FC04 | UInt32 | 0.1 | — | No |
| 406 | Extended Register 53 | FC04 | UInt32 | 0.1 | — | No |
| 408 | Extended Register 54 | FC04 | UInt32 | 0.1 | — | No |
| 410 | Extended Register 55 | FC04 | UInt32 | 0.1 | — | No |
| 412 | Extended Register 56 | FC04 | UInt32 | 0.1 | — | No |
| 414 | Extended Register 57 | FC04 | UInt32 | 0.1 | — | No |
| 416 | Extended Register 58 | FC04 | UInt32 | 0.1 | — | No |
| 418 | Extended Register 59 | FC04 | UInt32 | 0.1 | — | No |
| 420 | Extended Register 60 | FC04 | UInt32 | 0.1 | — | No |
| 422 | Extended Register 61 | FC04 | UInt32 | 0.1 | — | No |
| 424 | Extended Register 62 | FC04 | UInt32 | 0.1 | — | No |

9. Growatt MIN Series: Single-Phase Register Map (FC04/FC03 - 25 Registers)

| Address | Parameter Name | Function Code | Data Type | Scale | Unit | Plotted |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | Inverter Status | FC04 | UInt16 | 1.0 | — | Yes |
| 1 | PV1 Voltage | FC04 | UInt16 | 0.1 | VDC | Yes |
| 2 | PV1 Current | FC04 | UInt16 | 0.1 | A | Yes |
| 3 | PV1 Power | FC04 | UInt32 | 0.1 | W | Yes |
| 5 | PV2 Voltage | FC04 | UInt16 | 0.1 | VDC | No |
| 6 | PV2 Current | FC04 | UInt16 | 0.1 | A | No |
| 7 | PV2 Power | FC04 | UInt32 | 0.1 | W | No |
| 35 | AC Output Power | FC04 | UInt32 | 0.1 | W | Yes |
| 37 | Grid Voltage | FC04 | UInt16 | 0.1 | VAC | Yes |
| 38 | Grid Current | FC04 | UInt16 | 0.1 | A | Yes |
| 39 | Grid Frequency | FC04 | UInt16 | 0.01 | Hz | Yes |
| 40 | AC Output Voltage | FC04 | UInt16 | 0.1 | VAC | No |
| 41 | AC Output Current | FC04 | UInt16 | 0.1 | A | No |
| 53 | Energy Today | FC04 | UInt32 | 0.1 | kWh | Yes |
| 55 | Energy Total | FC04 | UInt32 | 0.1 | kWh | Yes |
| 57 | Work Time Total | FC04 | UInt32 | 0.5 | h | No |
| 93 | Inverter Temperature | FC04 | UInt16 | 0.1 | °C | Yes |
| 94 | IPM Temperature | FC04 | UInt16 | 0.1 | °C | No |
| 100 | Power Factor | FC04 | UInt16 | 0.001 | — | No |
| 105 | Fault Code | FC04 | UInt16 | 1.0 | — | No |
| 110 | Warning Bitmask | FC04 | UInt32 | 1.0 | — | No |
| 0 (H) | Remote On/Off Control | FC03 | UInt16 | 1.0 | — | No |
| 1 (H) | Active Power Rate % | FC03 | UInt16 | 1.0 | % | No |
| 22 (H) | Comm Address (Slave ID) | FC03 | UInt16 | 1.0 | — | No |